Thesis Title Page

Efficient Implementation of IEEE 802.11i Wi-Fi Security (WPA2-PSK) Standard Using

FPGA

A Thesis Submitted to the Graduate Faculty of

GRAND VALLEY STATE UNIVERSITY

In

Partial Fulfillment of the Requirements

For the Degree of

<name of your degree here>

<your academic unit>

<Month of graduation>  <Year of graduation>

# Acknowledgements

# Abstract

The rationale behind the thesis was to design efficient implementations of cryptography algorithms used for Wi-Fi Security as per IEEE 802.11i Wi-Fi Security (WPA2-PSK) standard. The focus was on software implementation of Password-Based Key Derivation Function 2 (PBKDF2) using Keyed-Hash Message Authentication Code (HMAC)-SHA1, which is used for authentication, and , hardware implementation of AES-256 cipher, which is used for data confidentiality.

In this thesis, PBKDF2 based on HMAC-SHA1 was implemented on software using C programming language, and, AES-256 was implemented on hardware using Verilog HDL. The overall implementation was designed and tested on Nexys4 FPGA board. The performance of the implementation was compared with other existing designs. Latency (us) was used as the performance metric for PBKDF2, whereas, throughput (Gb/s), resource utilization (Number of Slices), efficiency (GB/s per slice) and latency (ns) were used as performance metrics for AES-256. MRF24WG0MA PMOD Wi-Fi module was the 2.4 GHz Wi-Fi module which was interfaced with Nexys4 FPGA board for wireless communication.

When the correct security credentials were entered in the implemented system interfaced to the Wi-Fi module, it was successfully authenticated by a 2.4 GHz wireless router (or mobile hotspot) configured to work in WPA2-PSK security mode. Once this system was authenticated,  the implemented AES-256 cipher within the system was used to provide a layer of encryption over the data being communicated in the network.

# Table of Contents

Acknowledgements	2

Abstract	3

Table of Contents	4

List of Figures	5

List of Tables	6

Abbreviations	7

Chapter 1: Introduction	8

1.1	Background	8

1.2	Wireless Security Principle	9

1.3	WPA2-PSK	10

1.4	Project Scope	12

Chapter 2: Theory	13

2.1	 WPA2-PSK Device Authentication	13

2.1.1 	PBKDF2 (Password-Based Key Derivation Function 2)	15

2.1.2	 HMAC (Keyed-Hashing for Message Authentication)	18

2.1.3 	SHA1	19

2.2	WPA2-PSK Data Confidentiality	24

2.2.1	AES-256 Key Expansion	28

2.2.2	AES-256 Encryption	30

2.2.3 	AES-256 Decryption	30

Appendices	33

1.1	 AES Lookup Tables	33

References	37

# List of Figures

Figure 1: Example of Wi-Fi Network	9

Figure 2: WPA2-PSK Security	11

Figure 3: WPA2-PSK Authentication	11

Figure 4: WPA2-PSK Data Confidentiality	12

Figure 5: Block Diagram for PBKDF2	16

Figure 6: PBKDF2 with HMAC-SHA1	17

Figure 7: Block Diagram for HMAC	19

Figure 8: Block Diagram for SHA1 Processing Function	23

Figure 9: Input Bytes Arranged in State Array at Beginning of AES Operation	24

Figure 10: Output Bytes Arranged from State Array at End of AES Operation	25

Figure 11: Rcon( ) Values	25

Figure 12: Example of ShiftRows()	26

Figure 13: MixColumns() Calculation	27

Figure 14: Example if InvShiftRows()	27

Figure 15: InvMixColumns Calculation	28

Figure 16: Pseudo Code for Key Expansion for Encryption	29

Figure 17: Additional Pseudo Code to be Added for Key Expansion for Decryption	29

Figure 18: Pseudo Code for AES Encryption	30

Figure 19: Pseudo Code for the Equivalent Inverse Cipher	31

Figure 20: AES-256 Block Diagram	32

Figure 21: MicroBlaze Core Block Diagram	33

Figure 22: Layered Software Implementation of WPA2-PSK Authentication	35

Figure 23: Key BRAM  Organization	42

Figure 24: Data BRAM Organization	43

Figure 25: AES Core with Input and Output Signals	44

Figure 26: AES Module Interfaced with MicroBlaze	46

Figure 27: Implementation of S-Box in C	52

Figure 28: Implementation of Mul9 Lookup Table  in C	52

Figure 29: Implementation of Mul11 Lookup Table  in C	53

Figure 30: Implementation of Mul13 Lookup Table  in C	53

Figure 31: Implementation of Mul14 Lookup Table  in C	54

# List of Tables

Table 1: S-Box Lookup Table	33

Table 2: Inverse S-Box Lookup Table	33

Table 3: Mul2 Lookup Table	34

Table 4: Mul3 Lookup Table	34

Table 5: Mul9 Lookup Table	35

Table 6: Mul11 Lookup Table	35

Table 7: Mul13 Table	36

Table 8: Mul14 Table	36

# Abbreviations

# Chapter 1: Introduction

## Background

As most technologies have continued to transition from traditional wired systems to wireless ones, the number of wireless devices has grown by leaps and bounds over the last decade. Wireless devices have become a part of our day-to-day lives with its presence seen in household, educational and business institutions, to name a few. These devices are inter-connected with one another and share a variety of data, ranging from mundane to very personal and confidential information. Such interconnected devices that share data among themselves form a network. There can be various types of networks based on topology, size, area, organization, etc. One such type of network based on area is called Local Area Network (LAN). Such network is confined within a localized area such as a room, building or a group of buildings. However, it can be inter-connected to other LANs using wired or wireless media. If wireless medium is used to connect such LANs, then the overall network is called Wireless LAN (WLAN) [1].

The communication between the devices within a network is governed by a set of rules called communication protocols. The devices within a network must adhere to such protocols to successfully share and interpret data among other devices connected to the network. To maintain interoperability between the devices manufactured by various vendors, standardized communication protocols are defined for different type of networks.  One such protocol for communication between wireless devices over LAN is the IEEE 802.11 protocol and is commonly known as Wi-Fi [2]. An example of Wi-Fi network is shown in Figure 1.

Figure 1: Example of Wi-Fi Network

## Wireless Security Principle

Security is paramount in any type of network, but it is more so in the case of wireless networks, as they are far more vulnerable to attack in comparison to wired networks. In a wired network, the communicating devices must be physically connected using a cable. Hence, it is easier to verify the identity of the device to which the data is being communicated, as opposed to in wireless networks, where this is not quite easy. Also, unlike in wired networks, where the data is communicated through copper wires or optical fibers, in wireless networks, the wireless devices use RF signals in open air as their communication medium. So, theoretically any transceiver which is within the range of this RF signal and tuned to its frequency can read and/or meddle with the data being communicated [3]. Hence, for a secure communication, it is necessary to identity whether a device trying to connect to the network has proper security credential or not. This process is called authentication [3]. After a wireless device is authenticated to a network, the data being communicated within that network must be made confidential using a secure cryptography algorithm [3].

## WPA2-PSK Overview

The current standardized security protocol for Wi-Fi is IEEE 802.11i standard. This is also commonly known as Wi-Fi Protected Access II (WPA2). WPA2 was launched in September 2004 and supports PSK technology and includes an advanced encryption mechanism using the Counter-Mode/CBC-MAC Protocol (CCMP) called the Advanced Encryption Standard (AES) [4]. The PSK technology (in personal networks) is used to verify the identity of the communicating wireless devices. In PSK, the authentication process is performed by the access point (wireless router, mobile hotspot, etc.). With PSK, we can configure the access point (wireless router or hotspot) with a passphrase of 8 to 63 printable ASCII characters [5]. Using a technology called PBKDF2, that passphrase, along with the network SSID, is used to generate unique encryption keys for wireless clients. In WPA2-PSK security, the same set of SSID and PSK is shared between all Wi-Fi end devices and the access point as shown in Figure 2 [6]. The SSID is analogous to Username and PSK is analogous to Passphrase in Figure 2. The wireless devices are authenticated and granted access to the network, if the password to the particular  SSID matches [5]. After authentication,  AES cipher is used to maintain the confidentiality of the data being communicated within the network.

Figure 2: WPA2-PSK Security

In Figure 3, SSID and Passphrase goes through PBKDF2 to derive the 256-bit PMK which is used as the main key for AES cipher. The validity of this key is confirmed using the 4-way handshake process (Figure 2) between the Wi-Fi device and the access point [6]. If the key matches, then, the Wi-Fi device is successfully authenticated by the access point.

Figure 3: WPA2-PSK Authentication

After successful authentication, the data between Wi-Fi end devices and the access point is encrypted using AES cipher with the 256-bit PMK as the main key (Figure 4).

Figure 4: WPA2-PSK Data Confidentiality

## Project Scope

The purpose of this thesis is to optimize the cryptography algorithms used in device authentication and data confidentiality in Wi-Fi networks configured with WPA2-PSK security. To achieve this, the main key derivation part of the authentication process, as well as, the AES cipher algorithm required for data confidentiality will be optimized. The scope of the implementation will encompass the following areas:

- Efficient software implementation of PBKDF2 based on HMAC-SHA1 which is used for device authentication.

- Efficient hardware implementation of AES-256 cipher which is used for data confidentiality.

The performance of these implementations will be compared with other existing designs. Latency (us) will be used as the performance metric for PBKDF2, whereas, throughput (Gb/s), resource utilization (Number of Slices), efficiency (GB/s per slice) and latency (ns) will be used as performance metrics for AES-256.

# Chapter 2: WPA2-PSK Theory

## 2.1	 WPA2-PSK Device Authentication

Authentication is the process by which you prove that you are eligible to join a network (and that the network is legitimate) [3]. Pre-Shared Key (PSK) is a device authentication method used in WPA2-PSK networks, and it uses a passphrase of 8 to 63 printable ASCII characters to generate unique encryption keys [5].  The general idea of PSK mode is to use the same secret key on an access point and on a Wi-Fi device to authenticate the device and establish an encrypted connection for networking [6]. Hence, both Wi-Fi device and access point must prove to each other that they know the pre-shared key to ensure a secure connection. In WPA2-PSK, the access point (wireless router, hotspot, etc.) with a network SSID is configured with a passphrase. Using PBKDF2, that passphrase along with network SSID is used to generate the 256-bit Pairwise-Master-Key (PMK).  The Wi-Fi device must also derive the same PMK using the same passphrase and SSID for the access point to authenticate the device.

PMKs are never transmitted across the network as the channel of communication is not secure before the authentication process has completed. Because, without authentication, sharing of PMK would be done through an unencrypted channel and susceptible to be discovered by outside parties [7]. To overcome this, WPA2-PSK uses 4-way handshake to verify whether the Wi-Fi device and the access point have the same PMK or not (Figure 2). The 4-way handshake is designed so that the access point and Wi-Fi device can independently prove to each other that they know the PMK, without ever disclosing it. In Figure 2, the 4-way handshake is broken down into 4 messages [7]:

- Message 1 (From Access Point to Wi-Fi Device): The first step is for the access point to generate a nonce value. The nonce value is a pseudo random value generated by a publicly known and repeatable process. This pseudo random value is generated by the Pseudo Random Function 256 or PRF-256, as defined by WPA2 specifications. The nonce value generated by access point is called A-nonce. The access point sends a message containing this A-nonce value to the Wi-Fi device.

- Message  2 (From Wi-Fi Device to Access Point): The Wi-Fi device generates a nonce value using the same process as the access point and it is denoted as S-nonce. When the Wi-Fi device receives Message 1, it will generate Pairwise-Transient-Key (PTK). This key is required to be generated by both parties, and allows each party to verify that the other has the correct PMK. The creation of PTK is performed via another Pseudo Random Function (PRF), which uses a combination of the PMK, Access Point  MAC Address, Wi-Fi Device MAC Address, A-nonce and S-nonce [8]. A part of the PTK is known as the message integrity check (MIC). This value, along with the S-Nonce is then transmitted back to the access point.

- Message 3 (From Access Point to Wi-Fi Device): When the access point receives Message 2, it has all the values required to generate the PTK. The access point then generates the PTK, and checks whether the MIC value in Message 2 matches the MIC value that it has just generated. If the two MIC values matches, this proves that the Wi-Fi device knows the value of the PMK. If the MIC value is correct, the access point, then sends Message 3 to the Wi-Fi device. Message 3 allows the Wi-Fi device to ensure that the access point is a trusted party. If the access point did not have a matching PMK, the MIC would be different. Message 3 also informs the Wi-Fi device that the communication channel is about to be encrypted.

- Message 4 (From Wi-Fi Device to Access Point): The final part of the handshake allows the Wi-Fi device to acknowledge that the access point is now going to use encryption for the communication. After the Wi-Fi device transmits Message 4, it will install the encryption keys on the channel. After the access point receives message 4, it will install the encryption keys as well. All further unicast communication is protected by this encryption, until the client disconnects from the access point [3].

### 2.1.1 	PBKDF2 (Password-Based Key Derivation Function 2)

Using PBKDF2, the passphrase and SSID are hashed 4096 times to produce a 256-bit PMK [9]. Internally, the PBKDF2 key derivation function employed in WPA2-PSK utilizes 4096 iterations of HMAC-SHA1 to obtain 160-bit hash outputs. Since the PMK in WPA2-PSK is of 256-bits, two rounds of PBKDF2 are necessary [10]. Their outputs are concatenated, but for the second iteration the output is truncated to 96 bits to achieve the 256-bit PMK.  The PBKDF2 key derivation function is defined as follow:

DK = PBKDF2(PRF, P, S, C, dkLen) ................... (1)

where,

DK: Derived key

PRF: Pseudorandom function of two parameters with output length hLen

P: Password

S: Salt (sequence of bits)

C: Iteration count, a positive integer

dkLen: Length of Derived key

To derive key from PBKDF2, each hLen  bit block Ti of derived key DK, is computed as follows:

DK = T1 || T2 || ... || Tdklen/hlen ..................... (2)

Ti = F (P, S, C, i) ...................... (3)

In equation (3), the function F is the Exclusive-OR operations of C iterations of PRFs (as shown in equation (4)). In the first iteration, the PRF uses Password as the key and Salt concatenated with i (encoded as a big-endian 32-bit integer) as the 2 parameters (as shown in equation (5)). For, subsequent iterations, PRF uses Password as the key and the output of the previous PRF computation as the salt (as shown in equations (6) and (7)). The block diagram for PBKDF2 key derivation function is shown is Figure 5.

F (Password, Salt, C, i) = U1 ⊕ U2 ⊕ ... ⊕ Uc .... (4)

where,

U1 = PRF (Password, Salt || INT_32_BE(i)) ......... (5)

U2 = PRF (Password, U1) ........................... (6)

...

Uc = PRF (Password, Uc - 1) ......................... (7)

Figure 5: Block Diagram for PBKDF2

The overview of PBKDF2 along with HMAC and SHA1 is shown in Figure 6 [11].

Figure 6: PBKDF2 with HMAC-SHA1

In case of WPA2-PSK, the output and parameters in equation (1) are as follows:

PMK = PBKDF2(HMAC−SHA1, passphrase, ssid, 4096, 256) ...... (8)

### 2.1.2	 HMAC (Keyed-Hashing for Message Authentication)

HMAC provides a mechanism to calculate a message authentication code (MAC) based around a cryptographic hashing function [7]. A message authentication code (MAC) is a short piece of information used to authenticate a message. MACs are used between two parties that share a secret key to validate information transferred between them [12]. The use of HMAC in PBKDF2 is shown in Figure 6. The definition of HMAC requires a cryptographic hash function denoted by H, the secret key denoted by M and the message to be authenticated denoted by m. The HMAC function is defined as follows:

HMAC (M, m) = H ((M' ⊕ opad) || H ((M' ⊕ ipad)|| m)) ..........(9)

where,

H: a cryptographic hash function

M: the secret key

m: the message to be authenticated

M': another secret key, derived from the original key K

(by padding K to the right with extra zeroes to the

input block size of the hash function, or by hashing K if it is longer than that block size)

opad: the outer padding (0x5c5c5c…5c5c, one-block- long hexadecimal constant

ipad: the inner padding (0x363636…3636, one-block- long hexadecimal constant).

The block diagram for HMAC function is shown in Figure 7.

Figure 7: Block Diagram for HMAC

For WPA2-PSK, the parameters in equation (9) are as follows:

HMAC-SHA1 (passphrase, ssid) = SHA1 ((passphrase ⊕ opad)

|| SHA1((passphrase ⊕ opad ⊕ ipad)

||ssid) ... (10)

### 2.1.3 	SHA1

Hashing algorithms are used to process a message and produce a condensed representation of the message which is called a message digest, and for a perfect hashing function, it should be only one-way and a unique digital signature of the message [7]. The use of SHA1 in PBKDF2 is shown in Figure 6. The SHA1 hashing algorithm is valid for messages with a size less than 264 bits, it operates on blocks of size 512 bits, it uses a word size of 32 bits, and has a resultant message digest of 160 bits [7]. SHA1 algorithm primarily consists of 6 steps [13]:

Step1: Append Padding Bits: The original message is padded based on the following rules:

- The original message is first padded with one bit ‘1’.

- Zeros ‘0’ are then padded to bring the length of message to 64 bits less than multiple of 512.

Step2: Append Length: A 64-bit value indicating the length of the original message is appended to end the message obtained from Step 1 based on the following rules:

- 64-bit value of the original message is appended at the end of the padded message. If overflow occurs, the lower order of the 64-bit value is appended.

- The lower 32-bit word of the 64-bit value is appended first followed by the upper 32-bit value.

Step3: Prepare Processing Functions: SHA1 has 80 processing rounds. There are 4 mathematical operations assigned to each of the 4 sets of 20 rounds. These operations are as follows:

for 0 <= r <= 19,

F (r: B, C, D) = (B & C) | ((! B) & D) .............(11)

for 20 <= r <= 39,

F (r: B, C, D) = B ⊕ C ⊕ D................(12)

for 40 <= r <= 59,

F (r: B, C, D) = (B & C) | (B & D) | (C & D) .......(13)

for 60 <= r <= 79,

F (r: B, C, D) = B ⊕ C ⊕ D .....................(14)

Step4: Prepare Processing Constants: SHA1 has 4 different constants assigned to 4 sets of 20 rounds. These constants are as follows:

for 0 <= r <= 19,

K(r) = 0x5A827999 ............................... (15)

for 20 <= r <= 39,

K(r) = 0x6ED9EBA1 ............................... (16)

for 40 <= r <= 59,

K(r) = 0x8F1BBCDC ............................... (17)

for, 60 <= r <= 79

K(r) = 0xCA62C1D6 ............................... (18)

Step5: Initialize Buffer: SHA1 has five 32-bit buffers which are initialized as follows:

H0 = 0x67452301 ................................. (19)

H1 = 0xEFCDAB89 ................................. (20)

H2 = 0x98BADCFE ................................. (21)

H3 = 0x10325476 ................................. (22)

H4 = 0xC3D2E1F0 ................................. (23)

Step6: Process 512-bit block messages: The algorithm to process this 512-bit block of message is as follows:

For loop on k = 1 to N 	/* 1st For loop */

(W (0), W (1) ..., W (15)) = M[k] /*Divide M[k] into 16 words*/

For t = 16 to 79 do: /* 2nd For loop */

W(t) = (W(t-3) XOR W(t-8) XOR W(t-14) XOR W (t-16)) << 1

End of For loop /* 2nd For loop */

A = H0, B = H1, C = H2, D = H3, E = H4

For t = 0 to 79 do: // 3rd for loop

TEMP = A<<5 + f (t: B, C, D) + E + W(t) + K(t)

E = D, D = C, C = B<<30, B = A, A = TEMP

End of For loop // 3rd for loop

H0 = H0 + A, H1 = H1 + B, H2 = H2 + C, H3 = H3 + D,

H4 = H4 + E

End of for loop /* End of 1st For loop */

Output = H0 << 128 | H1 << 96 | H2 << 64 | H3 << 32 | H4

The block diagram for SHA1 processing function is given in Figure 8.

Figure 8: Block Diagram for SHA1 Processing Function

## 2.2	WPA2-PSK Data Confidentiality

WPA2-PSK uses Advanced Encryption Standard (AES) cipher for data confidentiality. The AES algorithm is a symmetric block cipher that can encrypt and decrypt information. Encryption converts data to an unintelligible form called ciphertext and decryption converts the ciphertext back into its original form, called plaintext [14]. AES has input block size of 128 bits and key size can be of 128, 192 or 256 bits. WPA2-PSK uses key size of 256 bits i.e. AES-256. It requires 60 rounds for key expansion and the size of the expanded key is 240 bytes. When decryption is performed using Equivalent Inverse Cipher method, then there are separate set of expanded keys for encryption and decryption processes [14]. Hence, in total, there will be 480 bytes of expanded keys when decryption is performed using Equivalent Inverse Cipher method. AES-256 requires 14 rounds each for the completion of encryption and decryption processes and the size of output block is 128 bits.

AES algorithm’s operations are performed on a two-dimensional array of bytes called the State [14].  For AES-256, the State consists of four rows of bytes, each containing 4 bytes.  At the start of the encryption and decryption, the input – the array of bytes In0, In1, … In15 – is copied into the State array as illustrated in Figure 9. The encryption and decryption operations are then conducted on this State array, after which its final value is copied to the output – the array of bytes Out0, Out1, … Out15 as shown in Figure 10. [14].

Figure 9: Input Bytes Arranged in State Array at Beginning of AES Operation

Figure 10: Output Bytes Arranged from State Array at End of AES Operation

AES consists of key expansion, encryption and decryption processes. These processes are realized with the help of following transformations [14]:

- RotWord():This function takes 4 bytes as an argument. It performs circular left shift on the 4 input bytes.

Example: 1,2,3,4 to 2,3,4,1

- Rcon (): This function returns a 4-byte value based on Figure 11.

Figure 11: Rcon( ) Values

- AddRoundKey(): In the AddRoundKey() transformation, a Round Key is added to the state by a simple bitwise XOR operation.

- SubBytes():In this transformation, each byte of data is substituted with the corresponding value from the S-box lookup table (Table1 in Appendix 1.1).

- ShiftRows():This function arranges the bytes of the state in 4x4 matrix and performs byte-wise circular left shift. The order of the shift varies with rows. The shift operation is not performed for the first row. The second row is shifted by 1 byte, the third row is shifted by 2 bytes and the fourth row is shifted by 3 bytes. An example of shift row operation is shown in Figure 12.

Figure 12: Example of ShiftRows()

- MixColumns():The matrix obtained from the ShiftRows() operation goes through the multiplication over Galois Field (Figure 13) . The lookup tables required for multiplication over Galois Field in the MixColumns() operation are shown in Table 3 and Table 4 in Appendix 1.1.

Figure 13: MixColumns() Calculation

- InvSubBytes():In this transformation, each byte of data is substituted with the corresponding value from the inverse S-box table (Table2 in Appendix 1.1).

- InvShiftRows():This function arranges the bytes of the state in 4x4 matrix and performs byte-wise circular right shift. The order of the shift varies with rows. The shift operation is not performed for the first row. The second row is shifted by 1 byte, the third row is shifted by 2 bytes and the fourth row is shifted by 3 bytes. An example of shift row is shown in Figure 14 [15].

Figure 14: Example if InvShiftRows()

- InvMixColumns():The matrix obtained from the InvShiftRows() operation goes through the multiplication over Galois Field (Figure 15) . The lookup tables required for multiplication over Galois Field in the InvMixColumns() operation are shown in Table 5, Table 6, Table 7 and Table 8 in Appendix 1.1

Figure 15: InvMixColumns Calculation

### 2.2.1	AES-256 Key Expansion

The AES algorithm takes the Cipher Key and performs a Key Expansion routine to generate a key schedule. The Key Expansion generates a total of Nb (Nr + 1) words: the algorithm requires an initial set of Nb words, and each of the Nr rounds requires Nb words of key data [14]. The resulting key schedule consists of a linear array of 4-byte words, denoted [wi ], with i in the range 0< i< Nb(Nr + 1) [14].

In AES-256, Nb = 4, and Nr = 14, so the key expansion routine generates a total of 60 words (240 bytes). The key expansion routine runs for 14 rounds and generates 240 bytes of expanded key. Two different sets of 240 bytes of expanded keys are generated for encryption and decryption when the decryption is done using Equivalent Inverse Cipher method. The expansion of the input key into the key expansion routine  proceeds according to the pseudo code in Figure 16 [14]. For AES-256, Nb = 4 and Nr = 14.

Figure 16: Pseudo Code for Key Expansion for Encryption

For the Equivalent Inverse Cipher, the following pseudo code shown in Figure 17 must be added to the end of the pseudo code shown in Figure 16 [14]. For AES-256, Nb = 4 and Nr = 14.

Figure 17: Additional Pseudo Code to be Added for Key Expansion for Decryption

### 2.2.2	AES-256 Encryption

In each round of AES encryption, the cipher makes four different transformations to the block of data. The four transformations are: AddRoundKey (), SubBytes (), ShiftRows () and MixColumns (). The exception is the final round, which only has three transformations since it does not have the MixColumns()operation [15]. During these rounds, each block of data is depicted as 4 x 4 byte matrix and the key is divided into 4 x 4 byte matrix as well. Each round gets these matrices as an input and produces 4 x 4 byte state matrix as an output.

The pseudo code for AES encryption is shown in Figure 18. For AES-256, Nb = 4 and Nr = 14.

Figure 18: Pseudo Code for AES Encryption

### 2.2.3 	AES-256 Decryption

Like encryption, in each round of decryption, the cipher makes four different transformations to the block of data. The four transformations being: InvAddRoundKey (), InvSubBytes (), InvShiftRows () and InvMixColumns (). The exception is the final round, which only has three transformations since it does not have the InvMixColumns()operation [15]. Similarly, during these rounds, each block of data is arranged as 4 x 4 byte matrix and the key is also arranged as 4 x 4 byte matrix. But, the expanded keys used here are different to the ones used for encryption. Each round gets these matrices as an input and produces 4 x 4 byte state matrix as an output. Pseudo code for the Equivalent Inverse Cipher is shown in Figure 19 [14]. For AES-256, Nb = 4 and Nr = 14.

Figure 19: Pseudo Code for the Equivalent Inverse Cipher

The overall block diagram of AES-256 is shown in Figure 20.

Figure 20: AES-256 Block Diagram

# Chapter 3: Software Implementation

### 3.1	 Overview

Derivation of PMK using PBKDF2 for authentication, and key expansion for encryption and decryption for AES-256,  was implemented in software. All software development in this thesis was implemented using combination of C and C++ programming languages for the MicroBlaze™ embedded processor soft core. Code related to PBKDF2 and AES-256 key expansion was written in C, while the application code was written in C++.

### 3.2	 MicroBlaze™ Environment

The MicroBlaze™ embedded processor soft core is a reduced instruction set computer (RISC) optimized for implementation in Xilinx® Field Programmable Gate Arrays (FPGAs). Figure 21 shows a functional block diagram of the MicroBlaze core [17].

Figure 21: MicroBlaze Core Block Diagram

The MicroBlaze soft core processor is highly configurable, allowing you to select a specific set of features required by your design. The fixed feature set of the processor includes  [17].

- Thirty-two 32-bit general purpose registers

- 32-bit instruction word with three operands and two addressing modes

- 32-bit address bus

- Single issue pipeline

MicroBlaze core is organized as a Harvard architecture with separate bus interface units for data accesses and instruction accesses [18]. It does not separate between data accesses to I/O and memory (i.e. it uses memory mapped I/O). The processor has up to three interfaces for memory accesses [18]: Local Memory Bus (LMB), IBM’s On-chip Peripheral Bus (OPB), and Xilinx CacheLink (XCL). MicroBlaze also supports reset, interrupt, user exception, break and hardware exceptions. For interrupts, MicroBlaze supports only one external interrupt source (connecting to the Interrupt input port) [18]. If multiple interrupts are needed, an interrupt controller must be used to handle multiple interrupt requests to MicroBlaze. The stack convention used in MicroBlaze starts from a higher memory location and grows downward to lower memory locations when items are pushed onto a stack with a function call [18]. Items are popped off the stack the reverse order they were put on. Writing software to control the MicroBlaze processor must be done in C/C++ language [18].

### 3.3	 WPA2-PSK Device Authentication

The code for WPA2-PSK authentication was written using C programming language. The overall code implementation contained 4 layers. At the bottom layer, there was code for SHA1 Hash algorithm. The second layer of code was for HMAC-SHA1, which called functions from the first layer. The third layer of code was for PBKDF2 which would call functions from the second layer. Finally, the application layer called the functions in the PBKDF2 layer for the complete WPA2-PSK functionality.  This layered architecture is illustrated in Figure 22. This code was written for 32-bit soft processor MicroBlaze.

Figure 22: Layered Software Implementation of WPA2-PSK Authentication

#### 3.3.1 	Layer1: SHA1-HASH Implementation

This layer dealt with all the functions related to the implementation of SHA1 Hash algorithm. The information regarding relevant data types and the function prototypes for all the functions in this layer are given below:

- Data Type: SHA1_CTX

This data type was used to maintain information relevant to an iteration of an SHA1-Hash process. It held information regarding 512-bits of input block, 160-bits of output hash, data length and bit length. This data type was used to pass information to and store information from all the functions defined in the SHA1-Hash Layer. The actual C code definition for this data type is given below:

typedef struct {

uint8_t data [64];

uint32_t datalen;

unsigned long long bitlen;

uint32_t state [5];

} SHA1_CTX;

- Function Prototype 1: SHA1Init()

void SHA1Init (SHA1_CTX *context)

This Function was used to initialize a new context for the SHA1-Hash process. It initialized datalen and bitlen to 0. It also initialized the states of the context to the initial Hash values of 0x67452301 ,0xEFCDAB89, 0x98BADCFE,

0x10325476 and 0xC3D2E1F0.

- Function Prototype 2: SHA1Update()

void SHA1Update (SHA1_CTX *context, const void *data, uint32_t len)

This function was used to update data, data length and bit length for the context of SHA1. If the data length was 64 bytes (512 bits) i.e. input block size of SHA1, we started the SHA1-Hash process by calling SHA1Transform().

- Function Prototype 3: SHA1Final()

void SHA1Final (unsigned char digest [20], SHA1_CTX* context)

This was the function where the initial processing before the actual SHA1-Hash processing was done. The initial padding and the appending of the data length to the input block was done in this function. After the initial processing, it called SHA1Transform () to perform the SHA1-Hash algorithm.

- Function Prototype 4: SHA1Transform()

SHA1Transform (SHA1_CTX *ctx, const uint8_t data [])

This was the main function where the processing part of the actual SHA1-Hash Algorithm was implemented. First, the 32-bit words of the 512-bit input block were stored into initial 16 arrays of size 32-bits. W (16) to W (79) values were then calculated from these values. 80 rounds of SHA1- Hash transform was performed on the data to get the 160-bits of SHA1-Hash Output value. Finally, the new Hash for the context was updated with the new output Hash value.

#### 3.3.2 	Layer2: HMAC_SHA1 Implementation

This layer dealt with the function related to the implementation of HMAC-SHA1. It contained a single function that called functions defined in SHA1-Hash algorithm. The prototype for this function is given below:

void my_hmac_sha1(const unsigned char *text,

int text_len, const unsigned char *key,

int key_len, unsigned char *digest)

This single function was used for the HMAC operation over SHA1-Hash. The first step of the code performed the initial padding and appending operations required for HMAC operation. After this initial process, this function would successively call SHA1Init (). SHA1Update (), SHA1Final () and SHA1Transform ().

#### 3.3.3 	Layer3: PBKDF2 Implementation

This layer dealt with the function related to the implementation of PBKDF2 operation. It contained a single function that called my_hmac_sha1() for 4096 iterations after some initial processing. The prototype for this function is given below:

int pkcs5_pbkdf2(const char *pass, const uint8_t *salt,

size_t salt_len, unsigned int rounds, uint8_t *key, size_t key_len)

256-bit PMK was derived from this function which was used as the first key in AES encryption. The application code would use this function to obtain the PMK from SSID-Passphrase combination, which is used to authenticate a wireless device using WPA2-PSK before starting the wireless communication.

### 3.4 	AES-256 Key Expansion

The code for AES-256 key expansion was written using C programming language and  for 32-bit soft processor MicroBlaze. The information regarding relevant function definitions for all the functions used for key expansion are given below:

- Function Definition 1: RotWord()

This function took an array which contained 4 one-byte values [a0,a1,a2,a3] as input, performed a cyclic permutation, and returned [a1,a2,a3,a0] as the output.

- Function Definition 2: SubWord()

This function took an array which contained 4 one-byte values [a0,a1,a2,a3] as input and returned S-Box substituted values of the array as the output. The code snippet for S-Box lookup table implementation in C is shown in Figure 23 (Appendix1.2).

- Function Definition 3: InvMixColumns()

This function took an array containing 16 one-byte values [a0,a1,a2 … a15] and number of bytes of completed expanded key for equivalent inverse cipher as an input, performed Inverse Mix Column transformations, and returned the transformed values and new count of the completed expanded key as the output. In this function definition , state was a global 4x4 one-byte array. The code snippets for mul9, mul11, mul13 and  mul14 lookup tables implementations in C are shown in Figure 24, Figure 25, Figure 26 and Figure 27 respectively in Appendix1.2.

- Function Definition 4: KeyExpansion()

This function took an array which contained 32 bytes of PMK as input, expanded it to 240 bytes each of expanded key for encryption and decryption, and returned these expanded keys as the output.

# Chapter 4: Hardware Implementation

### 4.1	 Overview

Encryption and decryption on data was performed using the designed AES-256 module. This module was implemented on Verilog HDL for Nexys4 FPGA board. The Verilog Code was written on Xilinx Vivado Design Suite 2017.2 IDE.

### 4.2	 Nexys4

Nexys4 board is a development platform based on the latest Artix-7™ (Xilinx part number XC7A100T-1CSG324C) Field Programmable Gate Array (FPGA) from Xilinx [19]. The Artix-7 FPGA is designed for high performance and it features 15850 logic slices (each with 6-input LUTs and 8 flip-flops), 240 DSP slices and 4860 KB of fast block RAM [20]. Nexys4 has generous external memories, and collection of USB, Ethernet, and other ports, and can host designs ranging from introductory combinational circuits to powerful embedded processors [19]. It also has several built-in peripherals such as an accelerometer, temperature sensor, MEMs digital microphone, a speaker amplifier, and a lot of I/O devices [19].

Figure 23: Nexys4 Board Features

### 4.3	 AES-256 Implementation

This hardware implementation of AES-256 primarily contained 2 modules: BRAMs (Key BRAM and Data BRAM) and AES core. BRAMs were used to store expanded keys, plaintext and ciphertext ,whereas, the AES core had modules for encryption and decryption blocks that used the expanded key, plaintext and ciphertext for encryption and decryption.

Since block size of AES is 128 bits (16 Bytes), both encryption and decryption processes worked on 128-bits (16 Bytes) of data at a time. If the data to be transformed was less than 128 bits, then, it was padded with trailing 0’s to make it 128-bit block before being transformed. If the data to be transformed was greater than 128 bits but not a multiple of 128 bits, then, it was also padded with trailing 0’s until we had a data block which is multiple of 128 bits. After padding, the encryption/decryption was done on one 128-bit block of data at a time. The data to be transformed was initially stored in a certain location of Data BRAM. At the beginning of the transformation, they are read from the Data BRAM and passed to the AES core. After the transformation was completed, the converted data was stored in a separate location of the Data BRAM which was allocated for the transformed data.

#### 4.3.1	 BRAMs

BRAMs are blocks of 32-bit memory locations used to store expanded key, original data to be encrypted or decrypted, and transformed data after encryption or decryption . It is a synchronous memory block with 32-bit data being clocked in or out at every clock. An instance of BRAM IP is shown in Figure 24. Two instances of BRAMs were created for the implementation of AES-256 module: Key BRAM and Data BRAM.

Figure 24: BRAM Module

- Key BRAM: The Data width of the Key BRAM was 32-bits. The expanded keys were stored in Key BRAM. 4 Bytes of expanded keys were stored per BRAM location. Hence, 60 memory locations were used to store 240 bytes of expanded key for encryption. Similarly, further 60 memory locations were used to store the other 240 bytes of expanded key for decryption. These keys were only read once from the BRAM at the beginning of the encryption/decryption process and saved into a temporary buffer. For multiple blocks of encryption/decryption, the keys were accessed from the temporary buffer instead of BRAM to improve the performance. The start address for Key BRAM was 0xC2000000. The overall memory organization for Key BRAM locations used in the design of AES-256 module is shown in Figure.

Figure 25: Key BRAM  Organization

- Data BRAM: It is a Read/Write Memory block where original data and data to be transformed are stored. The Data width of the Data BRAM is 32-bits. 16 such memory locations were allocated each for original data and transformed data. Hence, 128 bytes of BRAM memory was used for storing original and transformed data. The start address for Data BRAM was 0xC0000000. The overall memory organization for Data BRAM is shown in Figure.

Figure 26: Data BRAM Organization

#### 4.3.2	 AES Core

The block diagram of AES-256 core implementation is shown in Figure 27. It has the following port definitions:

Figure 27: AES Core with Input and Output Signals

- Input Ports:

  - Clk: It is the clock source to the AES core. The frequency of clock used in the design is 100 Mhz.

  - Reset: It is the synchronous reset signal to the AES core. It works on active low logic. The reset signal was controlled by bit 11 of Slave register0 from the AXI BUS of MicroBlaze.

  - Start: It is a 2-bit wide input signal which is used to start the encryption or decryption signal. If Start = 2’b01, the AES core will perform the encryption operation, if Start = 2’b10, the AES core will perform the decryption operation. All other possible values of Start signal are DON’T CARE cases.

The Start signal was controlled by bits[1:0] of Slave register0 from the AXI BUS of MicroBlaze.

  - Length: It is a 9-bit input value which signifies the number of 32-bit input data( after zero padding to make it multiple of 128-bit ) to be either encrypted or decrypted. The Length signal was controlled by bits[10:2] of Slave register0 from the AXI BUS of MicroBlaze.

  - Key: It is a 32-bit input value which signifies the expanded key input to the AES core. This input port was connected to 32-bit Data output port of  Key BRAM.

  - Data In: It is a 32-bit input value which signifies the plaintext or ciphertext to be encrypted or decrypted respectively. This input port was connected to 32-bit Data output port of  Data BRAM.

- Output Ports

  - Key Address: It is a 32-bit output value which denotes the memory location of the Key BRAM from which the expanded key is to be retrieved during the encryption/decryption process. On reset the value of Key Address is 0. This output port was connected to 32-bit address input port of  Key BRAM.

  - Data Address: It is a 32-bit output value which denotes the memory location of the Data BRAM from which the expanded key is to be retrieved during the encryption/decryption process. On reset the value of Data Address is 0.

This output port was connected to 32-bit address input port of  Data BRAM.

  - Data Out: It is a 32-bit output value which signifies ciphertext  in case of encryption operation and plaintext in case of decryption operation. On reset the value of Data Out is 0. This output port was connected to 32-bit Data input port of  Data BRAM.

  - Write Enable: It is a 4-bit output signal that is used to enable the write operation of the ciphertext in case of encryption or plaintext in case of decryption to the allocated memory locations in the Data BRAM. On reset the value of Write Enable is 0. This output port was connected to 4-bit Write Enable input port of  Data BRAM.

  - Done: It is a 3-bit output signal that shows the completion of the AES operation. The value of Done signal becomes 3’b111 after the AES transformation is completed and the new data is written into the allocated memory locations in the Data BRAM. On reset the value of Done is 0. This output signal was connected to bits[2:0] of Slave register1 from the AXI BUS of MicroBlaze.

The overall block diagram for interfacing AES core with BRAMs and MicroBlaze processor is shown in Figure.

# Appendices

## 1.1	 AES Lookup Tables

Table 1: S-Box Lookup Table

Table 2: Inverse S-Box Lookup Table

Table 3: Mul2 Lookup Table

Table 4: Mul3 Lookup Table

Table 5: Mul9 Lookup Table

Table 6: Mul11 Lookup Table

Table 7: Mul13 Table

Table 8: Mul14 Table

## 1.2	 Code Snippets

Figure 30: Implementation of S-Box in C

Figure 31: Implementation of Mul9 Lookup Table  in C

Figure 32: Implementation of Mul11 Lookup Table  in C

Figure 33: Implementation of Mul13 Lookup Table  in C

Figure 34: Implementation of Mul14 Lookup Table  in C

# References

| [1] | M. Souppaya and K. Scarfone, Guidelines for Securing Wireless Local Area Networks (WLANs), NIST Special Publication 800-153, 2012. |
| --- | --- |
| [2] | S. Banerji and R. S. Chowdhury, On IEEE 802.11: Wireless LAN Technology, International Journal of Mobile Network Communications & Telematics (IJMNCT) , 2013. |
| [3] | J. Edney and W. A. Arbaugh, Real 802.11 Security: Wi-Fi Protected Access and 802.11i, Addison-Wesley, 2004. |
| [4] | Wi-Fi Alliance, Deploying Wi-Fi Protected Access, 2005. |
| [5] | Juniper Networks, "Understanding PSK Authentication," Juniper Networks, 16 October 2018. [Online]. Available: https://www.juniper.net/documentation/en_US/junos-space-apps/network-director3.0/topics/concept/wireless-wpa-psk-authentication.html. [Accessed 24 October 2018]. |
| [6] | V. Fadyushin, "Common WLAN Protection Mechanisms and their Flaws," 16 March 2016. [Online]. Available: https://hub.packtpub.com/common-wlan-protection-mechanisms-and-their-flaws/. [Accessed 24 October 2018]. |
| [7] | T. B. Johnson, "An FPGA architecture for the recovery of WPA/WPA2 keys," Graduate Theses and Dissertations, 2014. |
| [8] | "Understanding WPA/WPA2 Pre-Shared-Key Cracking," 27 December 2015. [Online]. Available: https://www.ins1gn1a.com/understanding-wpa-psk-cracking/. [Accessed 26 October 2018]. |
| [9] | O. Nakhila, A. Attiah, Y. Jin and C. Zou, "Parallel Active Dictionary Attack on WPA2-PSK Wi-Fi Networks". |
| [10] | M. Kammerstetter, M. Muellner, D. Burian, C. Kudera and W. Kastner, "Efficient High-Speed WPA2 Brute Force Attacks using Scalable Low-Cost FPGA Clustering," 2016. |
| [11] | A. Visconti, S. Bossi, H. Ragab and A. Calò, On the weaknesses of PBKDF2, 2015. |
| [12] | H. Krawczyk, B. M. and R. Canetti, RFC2104: HMAC: Keyed-Hashing for Message Authentication, 1997. |
| [13] | H. Yang, "SHA1 Message Digest Algorithm Overview," 2017. [Online]. Available: http://www.herongyang.com/Cryptography/SHA1-Message-Digest-Algorithm-Overview.html. [Accessed 27 October 2018]. |
| [14] | National Institute of Standards and Technology(NIST), Announcing the ADVANCED ENCRYPTION STANDARD (AES), Federal Information Processing Standards Publication 197, 2001. |
| [15] | A. Berent, Advanced Encryption Standard by Example. |
| [16] | J. Monttinen, The Security of Advanced Encryption Standard, 2015. |
| [17] | Xilinx, MicroBlaze Processor Reference Guide, 2016. |
| [18] | R. Jesman, F. M. Vallina and J. Saniie, MicroBlaze Tutorial Creating a Simple Embedded System and Adding Custom Peripherals Using Xilinx EDK Software Tools. |
| [19] | Digilent, Nexys 4™ FPGA Board Reference Manual, 2016. |
| [20] | Xilinx, 7 Series FPGAs Data Sheet: Overview, 2018. |